# ADJUSTMENT GUIDE

ND Filter is in front of the Photocell at all times except when the read button is pushed in the LO range.

R4 adjusts the top of the HI range (21).

R2 adjusts the bottom of the HI range (II)

R3 adjusts the top of the LO range(II)

R1 adjusts the bottom of the LO range(I0)

Note: R1 will be usually near at the end of its travel.

R5 adjusts the battery check reading. Set the pointer on the red mark at 2.70 VDC.

This is a semi spotmeter so a K factor of 1.0 is suggested.